Basic M35A2 Data

Engine

Converted to run on DIESEL fuel ONLY.

IDLE at 1000rpm or less for 5 minutes before driving after start up and for 5 minutes after driving before shut down to prevent damage to turbosupercharger bearings.

Oil

15w40 22 qt capacity (with filter change)

Filters

- Oil Filter (front left top of engine-two units)

LF509N Fleetguard
P20HD Baldwin
CW203MP Wix
1133 Napa

- Secondary Fuel Filter  (back left top corner of engine-two units)

PF902 Baldwin
5573262 AC Delco
FF110 Fleetguard
3512 Napa

P550540 Donaldson (cross for Baldwin from Donaldson website, not verified)

- Primary Fuel Filter  (font right bottom corner of engine-one unit)

PF906 Baldwin

P553261 Donaldson (cross  for Baldwin from Danaldson website, not verified)

- Air Filter

PA1620 Baldwin
7E2HP97V Dollinger
18265-P11-7551 Donaldson (does not show on Donaldson website, gives P606073 as Baldwin cross)
AF826 Fleetguard/Hastings
250DX8B Novo Corp

Brakes

Use DOT5 Silicone Brake Fluid Only!!

Air over Hydraulic.  MC access panel on floor in front of driver seat, cap has vent line attached.

Drain air tanks (two) daily.

Electrical

24 volt

Batteries – Two Exide 6TL Ordnance (Military) in Series

Gear Lube (Gear Oil)

Use GL-1 ONLY, use of GL-5 will cause premature wear (failure) of bushings in transmission/transfer/differential

GL5 with MT1 certification should be ok as the formulation is not supposed to attack yellow metal bearings…