I've got a 2" reciever on the M35A2C that the Air Force put on while it was a Myrtle Beach and then I went and put one on the M35A2 that I converted to a tanker.   The setup on the tanker is nicer (better diagonal bracing) and I plan to modify the Air Force setup to match at some point...   Anyway, I realize that I don't have a good photo on-line of either setup, so I'll go out and take some when the rain stops.   Meanwhile, you can take a peak at these two for a little tease:

http://www.garbee.net/~cabell/m35pages/M35A2C/bluegrass2004/deuce6.jpg

http://www.garbee.net/~cabell/m35pages/M35A2C/bluegrass2004/deuce11.jpg

I've got a few buddies that want me to camo paint my hog cooker so that it will match my truck (use the same pattern as a water buffalo) since I tow it with the duece when I cater...

9.00R16 - Michelin tube 16N9, with a 1-16 or 170-16 Michelin flap, valve stem on tube is # 570 (metal)

11.00R16 - Michelin tube 16P9 or 16P, with a 6M16 Michelin flap, valve stem on tube is # 1339 (metal)

You can also get the correct tube/valve style by asking for a "lowboy trailer radial 11 x 16 tube."

International Tube and Tire (ITT) has a tube that is a good replacement for the Michelin tube listed above.  It is a 9/10/11 R15-16 Radial Truck Tube, ITT Product Code 11430, with Stem TR75A, which is a Brass Stem, with a 90 degree bend that just comes to the inner edge of the Split Ring, or Combat Wheel ring, so it doesn't hang out where it can be damaged.

International Tube and Tire (ITT) also makes flaps to match and the one to use is the 8 x 16 radial Flaps.

ITT website is:

http://innertubes.net/default.htm